Module 2

Assignment 1

Objective

To compare TCP and UDP throughput using iPerf3 and analyze their performance.

Test Setup

Tool: iPerf3

Server IP: 192.168.1.61

Test Duration: 10 seconds

TCP Throughput Test

Server Command

iperf3 -s

Client Command

iperf3 -c 192.168.1.61

O/P

Connecting to host 192.168.1.61, port 5201

[  5] local 192.168.1.61 port 59551 connected to 192.168.1.61 port 5201

[ ID] Interval           Transfer     Bitrate

[  5]   0.00-1.01   sec  1.79 GBytes  15.3 Gbits/sec

[  5]   1.01-2.00   sec  1.75 GBytes  15.0 Gbits/sec

[  5]   2.00-3.01   sec  1.78 GBytes  15.2 Gbits/sec

[  5]   3.01-4.01   sec  1.16 GBytes  9.94 Gbits/sec

[  5]   4.01-5.01   sec  1.04 GBytes  8.88 Gbits/sec

[  5]   5.01-6.01   sec  1.08 GBytes  9.33 Gbits/sec

[  5]   6.01-7.01   sec   848 MBytes  7.12 Gbits/sec

[  5]   7.01-8.01   sec   598 MBytes  5.04 Gbits/sec

[  5]   8.01-9.01   sec   733 MBytes  6.11 Gbits/sec

[  5]   9.01-10.01  sec   664 MBytes  5.60 Gbits/sec

- - - - - - - - - - - - - - - - - - - - - - - - -

[ ID] Interval           Transfer     Bitrate

[  5]   0.00-10.01  sec  11.4 GBytes  9.76 Gbits/sec                  sender

[  5]   0.00-10.01  sec  11.4 GBytes  9.76 Gbits/sec                  receiver

iperf Done.

Result

| Parameter | Value |
| --- | --- |
| Protocol | TCP |
| Duration | 10 seconds |
| Data Transferred | 11.4 GBytes |
| Throughput | 9.76 Gbits/sec |

Observation

TCP test completed successfully.

Data transferred: 11.4 GBytes.

Average throughput: 9.76 Gbits/sec.

TCP provides reliable communication.

UDP Throughput Test

Client Command

iperf3 -c 192.168.1.61 -u -b 1G

O/P

Connecting to host 192.168.1.61, port 5201

[  5] local 192.168.1.61 port 60503 connected to 192.168.1.61 port 5201

[ ID] Interval           Transfer     Bitrate         Total Datagrams

[  5]   0.00-1.01   sec   119 MBytes   987 Mbits/sec  1901

[  5]   1.01-2.00   sec   118 MBytes  1000 Mbits/sec  1897

[  5]   2.00-3.01   sec   120 MBytes   999 Mbits/sec  1917

[  5]   3.01-4.01   sec   119 MBytes  1000 Mbits/sec  1903

[  5]   4.01-5.01   sec   119 MBytes  1.00 Gbits/sec  1909

[  5]   5.01-6.01   sec   119 MBytes  1000 Mbits/sec  1913

[  5]   6.01-7.01   sec   119 MBytes  1000 Mbits/sec  1902

[  5]   7.01-8.01   sec   119 MBytes  1.00 Gbits/sec  1910

[  5]   8.01-9.01   sec   119 MBytes  1000 Mbits/sec  1909

[  5]   9.01-10.00  sec   119 MBytes  1.00 Gbits/sec  1905

- - - - - - - - - - - - - - - - - - - - - - - - -

[ ID] Interval           Transfer     Bitrate         Jitter    Lost/Total Datagrams

[  5]   0.00-10.00  sec  1.16 GBytes   999 Mbits/sec  0.000 ms  0/19066 (0%)  sender

[  5]   0.00-10.01  sec  1.14 GBytes   977 Mbits/sec  0.026 ms  414/19066 (2.2%)  receiver

iperf Done.

Result

| Parameter | Value |
| --- | --- |
| Protocol | UDP |
| Duration | 10 seconds |
| Data Transferred | 1.16 GBytes |
| Throughput | 999 Mbits/sec (Sender), 977 Mbits/sec (Receiver) |
| Jitter | 0.026 ms |
| Packet Loss | 2.2% |

Observation

UDP test completed successfully.

Throughput: 999 Mbits/sec (Sender).

Jitter: 0.026 ms.

Packet loss: 2.2%.

UDP is faster but does not guarantee packet delivery.

TCP vs UDP Comparison

| Feature | TCP | UDP |
| --- | --- | --- |
| Throughput | 9.76 Gbits/sec | 999 Mbits/sec |
| Reliability | Reliable | Not Reliable |
| Packet Loss | No | 2.2% |
| Jitter | Not Applicable | 0.026 ms |
| Retransmissions | Yes | No |
| Best Use | File Transfer, Web Browsing | Video Calls, Streaming |

Explanation

1. Differences Observed

TCP is reliable because it checks whether all packets are delivered.

UDP is faster but does not check packet delivery.

2. Jitter

Jitter is the variation in packet arrival time. In this test, the jitter was 0.026 ms, which is very low.

3. Packet Loss

Packet loss means some packets did not reach the receiver. In this test, the packet loss was 2.2%.

4. Retransmissions

TCP retransmits lost packets to ensure reliable communication. UDP does not retransmit lost packets.

5. Practical Recommendations

Use TCP for file transfers, web browsing, and email.

Use UDP for video calls, online gaming, and live streaming.

Keep packet loss and jitter as low as possible for better performance.

Conclusion

The iPerf3 tests showed that TCP provides reliable communication, while UDP offers faster transmission with a small amount of packet loss. The choice between TCP and UDP depends on the application requirements.

Bonus Assignment - Enterprise QA Lab Validation Strategy

Objective

To design a validation strategy for testing network and wireless performance in an enterprise QA lab using TRex, IXChariot, and VeriWave.

Validation Strategy

1. TRex

Purpose:

Generate high-volume network traffic.

Perform load and stress testing.

Used In QA Lab:

Validate routers, switches, and firewalls under heavy traffic.

Check network performance before product release.

2. IXChariot

Purpose:

Measure application performance over the network.

Used In QA Lab:

Test web browsing, file transfer, VoIP, and video conferencing.

Measure throughput, latency, and response time from the user's perspective.

3. VeriWave

Purpose:

Test enterprise Wi-Fi performance.

Used In QA Lab:

Simulate hundreds or thousands of wireless clients.

Validate wireless controllers, access points, roaming, and Wi-Fi stability.

Tool Summary

| Tool | Purpose | Used For |
| --- | --- | --- |
| TRex | Generate high network traffic | Load and stress testing |
| IXChariot | Measure application performance | VoIP, Video, Web, File Transfer |
| VeriWave | Validate Wi-Fi performance | Wireless client simulation and roaming tests |

Conclusion

An enterprise QA lab uses TRex for traffic generation, IXChariot for application performance testing, and VeriWave for wireless validation. Together, these tools help ensure the network is reliable, performs well under load, and provides a good user experience before deployment.

Assignment 2

1. Which tool would you use first and why?

Answer: Wireshark

Reason:
Wireshark captures network packets and helps identify issues such as slow responses, packet loss, retransmissions, and network delays.

2. What information would you collect using each tool?

| Tool | Information Collected |
| --- | --- |
| Wireshark | Packet loss, retransmissions, latency, DNS, HTTP/HTTPS traffic |
| TCPDump | Captures packets from a Linux server and saves them for analysis |
| iPerf | Measures TCP/UDP throughput, bandwidth, jitter, and packet loss |

3. What are three possible root causes?

- High network latency causing slow web page loading.

- Packet loss or high jitter causing video calls to freeze.

- DNS resolution delays causing websites to take longer to open.

4. How would you conclude whether the issue is network-related or application-related?

  - Run an iPerf test to check network performance.

  - Use Wireshark to look for packet loss, retransmissions, or high latency.

  - If the network performance is good but the application is still slow, the problem is likely application-related.

  - If there is high latency, packet loss, or poor throughput, the problem is network-related.

Conclusion

Using Wireshark, TCPDump, and iPerf together helps identify whether the issue is caused by the network or the application. In this scenario, the tools help analyze packet flow, measure network performance, and find the root cause efficiently.

Assignment 3

TCP 3-Way Handshake

Client sends SYN.

Server replies with SYN-ACK.

Client sends ACK.

TCP connection established successfully.

DNS Query and Response

Client requested the IP address of a website.

DNS server returned the IP address successfully.

HTTPS Transaction

Client communicated with the server using HTTPS (port 443).

The communication was encrypted.

Retransmissions

No retransmissions or duplicate ACKs were observed.

Overall Health

TCP connection established successfully.

DNS resolution completed successfully.

HTTPS communication was successful.

No retransmissions observed.

Overall communication was healthy.

Assignment 4

Objective

To identify the most suitable network tool for different networking scenarios and justify the selection.

| Scenario | Recommended Tool | Reason |
| --- | --- | --- |
| Measure TCP throughput between two hosts | iPerf3 | Measures TCP and UDP throughput, bandwidth, packet loss, and jitter between two hosts. |
| Capture packets from a Linux server | TCPDump | Command-line tool used to capture network packets on Linux systems for troubleshooting. |
| Validate enterprise Wi-Fi performance | VeriWave | Simulates multiple wireless clients to test Wi-Fi throughput, roaming, scalability, and stability. |
| Generate high-volume traffic | TRex | Generates high-speed traffic to perform load and stress testing of network devices. |
| Analyze mirrored switch traffic | Wireshark | Captures and analyzes mirrored packets to identify network issues and inspect protocols. |

Conclusion

Each tool has a specific purpose:

iPerf3 - Measures network performance.

TCPDump - Captures packets on Linux.

VeriWave - Validates enterprise wireless networks.

TRex - Generates high network traffic.

Wireshark - Analyzes captured network packets.

Using the correct tool helps identify and resolve network issues efficiently.

Assignment 5

sample.py

import subprocess

import re

import sys

# Full path to iperf3.exe

IPERF_PATH = r"C:\Users\SARAVANA SELVAN\Downloads\iperf3.21_64\iperf3.21\iperf3.exe"

# Server details

SERVER_IP = "192.168.1.61"

DURATION = 10

try:

# Run the iPerf3 client

result = subprocess.run(

[IPERF_PATH, "-c", SERVER_IP, "-t", str(DURATION)],

capture_output=True,

text=True

)

output = result.stdout

# Print the complete iPerf3 output

print(output)

# Extract average bandwidth

bandwidth = 0

match = re.search(r'0\.00-\d+\.\d+\s+sec\s+[\d.]+\s+[GM]Bytes\s+([\d.]+)\s+([GM])bits/sec', output)

if match:

value = float(match.group(1))

unit = match.group(2)

if unit == "G":

bandwidth = value * 1000   # Convert Gbps to Mbps

else:

bandwidth = value

# Extract retransmissions (if available)

retransmissions = 0

retrans_match = re.search(r'(\d+)\s+sender', output)

if retrans_match:

retransmissions = retrans_match.group(1)

# Print summary

print("\n========== Test Summary ==========")

print(f"Average Bandwidth : {bandwidth:.2f} Mbps")

print(f"Total Retransmissions : {retransmissions}")

print(f"Test Duration : {DURATION} seconds")

# Pass/Fail

if bandwidth >= 800:

print("\nPASS : Bandwidth is acceptable.")

else:

print("\nFAIL : Bandwidth is below 800 Mbps.")

sys.exit(1)

except FileNotFoundError:

print("Error: iperf3.exe not found. Check the file path.")

except Exception as e:

print("Error:", e)

O/P

Connecting to host 192.168.1.61, port 5201

[  5] local 192.168.1.61 port 63409 connected to 192.168.1.61 port 5201

[ ID] Interval           Transfer     Bitrate

[  5]   0.00-1.01   sec  1.24 GBytes  10.5 Gbits/sec

[  5]   1.01-2.01   sec  1.24 GBytes  10.7 Gbits/sec

[  5]   2.01-3.00   sec  1.63 GBytes  14.1 Gbits/sec

[  5]   3.00-4.01   sec  1.86 GBytes  15.8 Gbits/sec

[  5]   4.01-5.01   sec  1.82 GBytes  15.7 Gbits/sec

[  5]   5.01-6.01   sec  1.40 GBytes  12.1 Gbits/sec

[  5]   6.01-7.00   sec  1.11 GBytes  9.58 Gbits/sec

[  5]   7.00-8.01   sec  1.56 GBytes  13.2 Gbits/sec

[  5]   8.01-9.01   sec  1.75 GBytes  15.1 Gbits/sec

[  5]   9.01-10.00  sec  1.75 GBytes  15.1 Gbits/sec

- - - - - - - - - - - - - - - - - - - - - - - - -

[ ID] Interval           Transfer     Bitrate

[  5]   0.00-10.00  sec  15.3 GBytes  13.2 Gbits/sec                  sender

[  5]   0.00-10.00  sec  15.3 GBytes  13.2 Gbits/sec                  receiver

iperf Done.

========== Test Summary ==========

Average Bandwidth : 10500.00 Mbps

Total Retransmissions : 0

Test Duration : 10 seconds

PASS : Bandwidth is acceptable.

Assignment 6

Validation Workflow

| Tool | Purpose | When to Use (Customer Environment) |
| --- | --- | --- |
| Wireshark | Capture and analyze network packets | When users report VoIP issues, slow web browsing, or packet loss. |
| Airtool | Capture and analyze wireless packets | When Wi-Fi clients experience roaming problems, weak signals, or interference. |
| Port Mirroring | Copy switch traffic for analysis | When traffic between the WLC, Access Points, and network devices needs to be monitored without affecting the network. |
| TCPDump | Capture packets on Linux-based devices | When debugging communication between servers and the wireless controller. |
| TRex | Generate high-volume traffic | Before firmware release, verify that the WLC can handle heavy traffic from many clients without performance degradation. |
| VeriWave | Simulate wireless clients | Simulate up to 1000 Wi-Fi clients to validate throughput, roaming, scalability, and controller stability. |
| iPerf3 | Measure TCP and UDP performance | Measure throughput, bandwidth, jitter, and packet loss after the firmware upgrade. |
| Multicast Hammer Tool | Generate multicast traffic | Validate IPTV, multicast video streaming, and multicast traffic handling by the controller. |
| Octoscope | Create controlled wireless test environments | Test firmware performance under different RF conditions such as interference, attenuation, and client roaming. |
| IXChariot | Measure application performance | Validate VoIP, video conferencing, web browsing, and file downloads from the end-user perspective after the firmware update. |

Conclusion

The firmware validation process uses multiple tools to test different aspects of the wireless network. VeriWave simulates a large number of Wi-Fi clients, TRex generates heavy network traffic, iPerf3 measures throughput, Wireshark and TCPDump analyze packets, and IXChariot validates application performance. Together, these tools help ensure that the Cisco Catalyst 9800 Wireless Controller performs reliably before deployment.